Debug/Reflection Log - Robotics Group 28

Ryan Barrett

Summary

This debug log documents the evolution of our robot control system, tracking both hardware and software issues encountered during development. A key discovery was our incorrect assumption about servo center positions, which led to mechanical alignment problems.

Total Entries: 6

Hardware Issues: 4

Software Issues: 2

Incorrect Assumptions Discovered: 1

---

## Entry 1 - 2026-01-30: Templates Folder Restructure

Type: SOFTWARE

File(s): `index.html`

Commit: 691c63a

Issue: Web application structure needed proper organization

Fix: Moved `index.html` to `templates/` folder for better Flask app structure

Impact: Web server can now properly serve templates using Flask conventions

---

## Entry 2 - 2026-02-02: Servo Speed Defaults Reduced

Type: HARDWARE

File(s): `Project_1/robot.py:46,58`

Commit: a6a8db3

Issue: Robot moving too fast with default speeds, causing instability

Fix:

- Reduced `driveForward()` default speed from 0.5 to 0.25

- Reduced `driveBackward()` default speed from 0.5 to 0.25

- Reduced `turnLeft()` default speed from 0.3 to 0.15

- Reduced `turnRight()` default speed from 0.3 to 0.15

Impact: More controlled and stable robot movement at default speeds

---

## Entry 3 - 2026-02-05: Motor Speed Limits Capped

Type: HARDWARE

File(s): `Project_1/robot.py:53,65,77,89`

Commit: a6a8db3

Issue: Maximum speed of 1.0 was too fast for safe operation

Fix: Changed speed limit from `min(1.0, speed)` to `min(0.75, speed)` across all drive functions

Impact: Robot now caps at 75% speed, preventing dangerous high-speed movements

---

## Entry 4 - 2026-02-02: Joystick Y-Axis Inversion Bug Fixed

Type: SOFTWARE

File(s): `Project_1/index.html:262`

Commit: 9351327

Issue: Joystick up/down direction was inverted - pushing up caused robot to go backward

Fix: Removed negation from `normalizedY` calculation: changed from `-y / maxRadius` to `y / maxRadius`

Comment Added: "FIXED: Flip Y axis so up is forward, down is backward"

Impact: Joystick controls now match intuitive expectations (up = forward, down = backward)

---

## Entry 5 - 2026-02-05: Waist Servo Center Position Corrected ⚠️ INCORRECT ASSUMPTION

Type: HARDWARE

File(s): `Project_1/robot.py:28-30`

Commit: 9351327

Issue: Waist servo default center position (0°) was mechanically incorrect

Incorrect Assumption: We initially assumed the servo's center position corresponded to 0° in our coordinate system, which is typical for many servo implementations. However, testing revealed the actual mechanical neutral position of our waist servo assembly is at -35°. This was likely due to how the servo horn was mounted on the physical assembly.

Fix:

- Set waist servo center to -35° instead of 0°

- Calculated position: `waist_center = 6000 - int((35.0 / 90.0) * 2000)`

- Updated Servo initialization to use `default_position=waist_center`

Note: Added comment "Center position is -35 degrees (not 0)"

Impact: Robot waist now centers at correct mechanical neutral position

Lesson Learned: Always physically verify servo zero positions after assembly, as mounting orientation affects the relationship between software angles and physical positions

---

## Entry 6 - 2026-02-05: Waist Rotation Range Limited

Type: HARDWARE

File(s): `Project_1/index.html:214`

Commit: 9351327

Issue: Full 90° rotation range exceeded mechanical limits

Fix: Changed waist rotation max from 90° to 20° in web interface slider

Range: Updated from `min="-90" max="90"` to `min="-90" max="20"`

Default Value: Changed from 0° to -35° to match center position

Impact: Prevents servo damage from exceeding mechanical rotation limits