Project 2 Debug Log

Entry 1 — 2026-02-23 | Project 2 Creation

Status: Initial setup complete

Branch: project-2 (merged into main)

What was built

•	
      dialog_engine.py — Parses the DSL script file, performs rule matching, returns (speak_text, [action_tags]). Tracks scope level and variables.

•	
      action_runner.py — Maps action tags (<head_yes>, <head_no>, <arm_raise>, <dance90>) to robot primitives. Runs actions from a background thread queue.

•	
      app.py — Extended from Project 1 with a /dialog POST endpoint accepting {"text": "user input"}.

•	
      Supporting files carried over from Project 1: robot.py, motor.py, servo.py, head.py, speaker.py, maestro.py, templates/

Entry 2 — 2026-03-11 | Arm Raise Watchdog Race Condition

Status: Documented — fix recommended before demo

Branch: main

What was tested

Triggered <arm_raise> via the dialog engine by saying "raise your arm" and "wave at me".

Issue found

_arm_raise() in action_runner.py has a total sleep time of exactly 4.0s (0.5 + 0.5 + 2.5 + 0.5). The watchdog cap (CAPS['arm_raise']) is also 4.0s. On slow hardware, the watchdog can fire before the final setArmAngle(0) / setElbowAngle(0) / setWristRotation(0) calls execute, leaving the arm stuck in the raised position.

Root cause

_sleep(2.5) at action_runner.py:149 pushes total runtime to the cap boundary.

Fix

Reduce _sleep(2.5) to _sleep(2.0) so total runtime is ~3.5s, giving a 0.5s safety buffer under the 4.0s cap.

Entry 3 — 2026-03-11 | Parameter Boundary Testing (/drive, /dialog)

Status: NaN issue documented; content-type issue low priority

Branch: main

What was tested

Sent edge-case values to the /drive and /dialog POST endpoints.

Issues found

1	
      NaN bypass in /drive: float('nan') passes the abs(speed) > 1.0 guard in app.py:68 — NaN comparisons always return False in Python, so nan slips through to motor.setSpeed() without rejection.

2	
      Vague error on missing Content-Type: Sending a request to /dialog without a Content-Type: application/json header causes request.get_json() to return None. The existing check catches it and returns "Missing text", but the error message gives no hint that the JSON body was never parsed.

Fix / Status

•	
      Add math.isnan() check in /drive after casting to float.

•	
      Content-type issue is low priority for demo; documented here for awareness.

Entry 4 — 2026-03-11 | Input File Reader: # in Patterns Silently Truncated

Status: Known DSL limitation — no impact on demo script

Branch: main

What was tested

Loaded modified versions of the dialog script with edge-case syntax to probe parser behavior.

Issue found

The parser strips comments using raw_line.split('#')[0] at dialog_engine.py:97. Any pattern containing a literal # character (e.g., u:(what is c#): ...) is silently truncated at the #. The resulting mangled line fails RULE_RE and is logged as "unrecognized line format" with no hint that # was the cause.

Root cause

split('#')[0] is applied unconditionally before any other parsing. There is no escape mechanism for # in the DSL.

Fix / Status

Document as a known DSL limitation. testDialogFileForPractice.txt does not use # inside patterns, so there is no impact on the demo.

Entry 5 — 2026-03-11 | Index Text Box: BOOT State UX Issue

Status: Low priority — cosmetic; no code change needed for demo

Branch: main

What was tested

Opened the page while the server was starting, and typed into the dialog input box before the script was loaded.

Issues found

1	
      When the engine is in BOOT state (no script loaded), the Send button is fully active. Submitting a message returns (no match) in the conversation log with no explanation that the engine isn't ready yet.

2	
      There is no visual indication that the state badge turning green (IDLE) is the signal that the dialog box is ready to use.

Fix / Status

sendDialog() in index.html could check stateBadge.textContent === 'BOOT' and display a warning instead of sending. Low priority — for the demo, simply load the script first via the "Load Script" button before typing.

Entry 6 — 2026-03-11 | Safety Interrupt: Partial-Word Inputs Not Caught

Status: Behavior matches spec — documented for demo awareness

Branch: main

What was tested

Typed safety words in various forms and casings into the dialog input box.

Findings

•	
      "stop", "STOP", "Stop." (punctuation is stripped before matching) all correctly trigger the safety interrupt and return "OK. Stopping now.".

•	
      "stop please", "please stop", and "oh stop" do not trigger the interrupt — the regex ^(stop|cancel|reset|quit)$ at dialog_engine.py:256 requires an exact, whole-input match.

Root cause

Intentional per the spec: the safety interrupt checks for the word alone on the input line.

Fix / Status

No fix needed — behavior matches spec. Note for the demo: the grader should type the interrupt word by itself (e.g., just stop) rather than in a sentence.